Schuldiner Lab Standard Operating Procedures 2025

Last updated: April 2025 (by Maya, Rosario, Hadar, Yeynit, Lior, Reut, Ofir, Sarah, Noga, Olga, Dunya, Mor, Din, Hanni & Terry)

Here is your guide to all the inside info and special tips that will make your work and life in the Schuldi lab easier and more productive! Everything is important and you should read it from cover to cover so that you’ll know how to just jump straight to your favorite part…

Enjoy and good luck!

Table of Contents

General Lab Stuff

Lab Culture and Life

Files and Electronic Storage

Plasmids

DNA Gels

Strains and Media

Transformations

General Yeast Growth

Singer Plates and Pads

SGA and Libraries

Western blots

Microscopy

Orders

# General Lab stuff

- Please use the following link to schedule any of our lab instruments/TC laminar hood, in our online booking system: https://prodis.weizmann.ac.il/pls/htmldb/f?p=404:101:8434321045640:::::

- Put dirty glassware, racks, dishes and other labware for autoclaving on the cart OUTSIDE the lab (not inside). If there is paper at the bottom of the trays this means they are clean.

- Please request specific agar plates for your own experiments from Reut, or ConA plates from Yeynit - at least five working days in advance (preferably one week…).

- Plates are usually not poured on Thursday since they must be wrapped a day after pouring. However, if you know that you will be in the lab on Friday, you can ask to make the media on Thursday and you will be responsible for wrapping them yourself.

- If you notice that there is only one sleeve of plates left in the general plates’ fridge, please write it in the media ordering Excel sheet. Same for the AA fridge, the antibiotics boxes and the general liquid media cabinet. For the antibiotics – let Reut know by email.

- If any reagent in the lab is about to run out or you take the -4 aliquot of something, please ask Hanni (using the Excel sheet) to order immediately so that we can buy a new one in time. Please: 1) Add the item to the ordering list in the Dropbox (also do so if you have sent an email to Hanni – it will help her to keep track), 2) Ask for quotes from the distributor for the actual price for Israel in shekels (unless it is from Sigma or the warehouse) and add it to the ordering folder in the Dropbox. 3) If the item is regularly ordered, you should find a list of different representatives in the lab, in the orders folder. If you can’t find it in there, check the general email in the Lab inventory file, in the Order folder of the Dropbox.

- When ordering a service through Hanni, tell her what it is and point out if you are getting just data or physical products as an output. For example, when ordering sequencing vs ordering a plasmid synthesized.

- If you have a contaminated media bottle, put it in the sink (or if it’s extremely contaminated – in the chemical hood) with 1/10 volume of 100% bleach for 30 min and only then empty it down the drain and rinse well, before placing the bottle on the dirty glassware cart OUTSIDE the lab. Set a timer so that you do not forget it in the sink and please make sure that your name is written on the bottle. If the contamination is too big to go down the sink, empty it into the bio-hazard bin. Afterwards rinse the sink until there is no longer a smell of bleach.

- When growing strains in glass tubes, please put a sticker only on the lid (with one side not stuck hard) – DO NOT put a sticker on the tube itself or write on it with a marker. It is also better to use the “not cryogenic/tough spots” stickers since they are easier to peel off when cleaning, and cheaper. If you still insist on doing other stuff – please remove all before lab cleaning.

- Please use the cheap Lifegene tips for vacuuming and the old robot LifeGene tips for picking and vacuuming (they are clearly marked in the cabinet). Sometimes we get expired robot tips or they don’t fit our pipettors - they are also used for picking.

- When you want to plan an experiment, write every step you will have to do and put it in your calendar. This way there is a less chance that you will forget to do something in the process. Always check that you have all the materials you need and prepare them in advance. Take into consideration that there are things that you will have to order in advance which may take time. Also check that technical instruments with their functions, options and limits are available and are suitable for your experiment. If, for any reason, your plans change, please update the relevant staff/calendars and take yourself off the booking page for the instruments.

- If you lend something from the lab to someone outside the lab, please notify Hadar immediately by email/WhatsApp. Write: your name/who took it/what did they take/ their email/ their phone number/ which lab are they from.

- If someone comes for a plasmid/strain/library, make sure they have already cleared it with Maya beforehand.

- When you have finished working in the fume hood, make sure to close it all the way and switch off the lights.

- Commonly used shared solutions are found in the gel room. When the solution reaches the “lower line” please dilute it and make a new batch (from bought concentrate). If you open the one-before-last bottle of concentrate – make sure to order a new one.

- Do not forget to put bleach in the trap of your vacuum pump.

- Please clean your trap once a month (at least). Do not wait for lab cleaning day. Check and verify that the secondary trap (under your bench, by the pump) is dry. If it is not, notify Hadar.

- Clean the tube of the trap once a week by sucking ethanol or 10% bleach through the tube thus preventing anything from growing in it.

- When you see anything that needs to be bought/ made/ fixed, notify the person in charge of it as soon as possible (if you are not sure who it is, you can always ask Hadar/Reut).

- When leaving- If you are the last person to leave the lab at the end of the day, or if you leave any time and there is no-one else in the lab, please ensure to lock all the doors when leaving the lab. It is your responsibility to make sure that our lab is safe! This includes the: 1) main door towards Meyer building (just bang it shut, it will lock automatically. If you turn the key, the mechanism for opening with the code will not work). The door between our lab and the kitchen (activate the lock by pushing the silver lock on the side of the door down), 3) from the kitchen towards outdoors (just close, locks automatically).

- At the end of the day, to help save energy and protect our planet, please turn off your computer screens, any non-working pieces of equipment, disconnect chargers from the wall and shut down all lights in rooms not used.

- If you use a protocol and see that it is not updated, things are missing or can be clearer – please update it with track changes, send to Maya for authorization and then save it into the protocol folder with the changes accepted AND a note at the beginning of the protocol saying when it was updated and by whom. Then move the old protocol into the “old protocols” folder and send an email notifying the lab that there is an updated version of the protocol. To help others, you may consider adding comments in the document regarding specific steps.

- If using a new protocol in the lab that may be relevant for others, send to Maya to see if it would be worth adding to general protocol lists.

- Make sure to take your biohazard bag to the main Biohazard bin twice a week – every Monday and every Thursday. The Biohazard bin should NEVER be left in the lab over the weekend as Drosophila contamination can occur. Small- and big biohazard bags can be found under the sink or in the main Biohazard room.

- Never throw food in the bins inside the lab or Eva’s room. Food kept in personal drawers should be in closed packaging, to avoid attracting ants.

- If you encounter a persistent problem in the lab, write it in the lab’s WhatsApp group for quicker resolution.

- We re-use intact and dry ziplock plastic bags for wrapping Singer plates and for chemical waste (stored under Reut’s desk). If they get torn, please put them in plastic recycling. We also reuse round plate plastic bags when people order less than a full sleeve from Reut.

- Before working with dangerous chemicals, prepare in advance a chemical bag labeled according to the chemical that you will be working with. Once you have finished working in the hood, put the bag in the chemical waste blue cart (in the same room as the biohazard red trash). If you are not sure which sticker to apply, check in the WeizmannNet app or this link: https://www.weizmann.ac.il/safety/chemical-sticker/. Do not leave your chemical waste in the hood! It will prevent proper ventilation and put others at risk. Residual liquids should be stored in an appropriate bottle, labeled with a sticker and disposed of in the same manner.

- If you need to learn how to perform a new lab technique (Planning primers, yeast growth, SGA, etc.) – check the file "Training responsibilities for new people joining the lab" for the appropriate person to help you out.

- If you wish to work with mammalian cells, read the TC-room SOP and protocols and get permission from Maya.

- When you order a chemical, add it to the chemical Excel with all the details.

- We have a lab desk-autoclave that can be used to autoclave only liquids, not glass or plastic ("Dry" autoclave) as there's no drying function (even though there are existing programs called "plastic" and "glass"). If someone wants to use it, please first ask Reut how to do so.

- Label the important things on your bench with your initials, as things disappear easily to other benches or rooms.

- Label your boxes (in 4ºC and -20ºC and Revco) with your name on them so they can be traced back to you. If you need more storage space, DO NOT randomly put your boxes where there is free space. Rather ask Hadar and get a set location so that we keep things organized.

- There are a numerous lab activities, lab tasks and meetings that are either mandatory or elective. All activities will be sent as invitations to your calendar.

- After giving a GM, check when your next GM will be and remind yourself two weeks in advance so that you start preparing in time. If your GM date changes, Hanni will let you know. When you finish giving your GM, please send your presentation to Hanni right away.

- Avoid working on other people’s benches. If you must work on someone else’s bench, ask for their permission first and then make sure to leave it EXACTLY as you found it and do not use their tips/pipettes/buffers etc.

- Do not take anything from other people’s benches without getting permission (pipettes, timers, buffers etc.). If you have to take something – put it back in the exact place and in the same (or better) condition than you found them, and let the person know.

- If you bring things with you for your work on a general bench space (DNA gel area, hoods, Revco area etc…) make sure to remove them and leave the area as you found it (i.e. clean and organized).

- If you finish some equipment (such as tips in the DNA gel area) or reagent (such as DDW/Buffers), make sure to replace them. (Not replacing the box because it still has one tip left, is not ok!)

- Respect other people in the lab by keeping the lab quiet – take loud conversations (meetings, phone calls) outside the lab.

- If you set a timer with an alarm, be sure to be around when it goes off or take it with you.

- If you need a quiet place to study/read/write, you can book any Mol Gen seminar room using Outlook, or you can order a room in the library. For more information: https://weizmann.libanswers.com/faq/300753. There are also a couple of rooms with internet connection from a wall socket that are optimal for zoom meetings (Meyer building M106, M218, M310, M403; Belfer building B225, B425.)

- When going on a planned vacation/ leave of any kind, first notify Hadar, Hanni and Maya. Make sure to leave someone in charge of your lab cleaning duty, everyday lab duties, and libraries you are maintaining.

- If you are responsible for bringing a birthday cake or preparing a gift and cannot make it, it is your responsibility to find a replacement and let Hanni know.

- You can get inspiration for a gift from the folder (office supply drawer next to Maya’s office) where we store good ideas for a birthday gift for every specific person. Feel free to use them or to add a note.

-Please recycle paper/industrial plastic/aluminum foil/plastic or glass bottles in the indicated bins in our lab. If one of these bins is full, inform Lior and Benjamin, who are in charge of recycling in the lab, and lend a hand to empty it.

Kitchen – Everyone appreciates if you bring food/drinks when coming back from abroad.

- Mark your food in the fridge with name and date. Put stuff in sealed boxes (not hamgashi’yot)

- We have a small grocery list hanging on our fridge where we can add items that should be bought for general use for the lab (i.e. salt, oil, condiments, tea, coffee etc.). Please use it only for such requests and not for personal food items.

- Please always wash your dishes immediately and leave the kitchen sink clean. Store opened snacks in one of the sealable boxes from the bottom drawer in the kitchen. Do not leave cups on your desk.

- Dirty towels should be put in the plastic bag on the shelf under the window.

- Please make sure the garbage can is closed hermetically.

- The right middle drawer in the kitchen is for kosher dishes. If you don't know what that means, don't use the drawer or the dishes and don’t put your dishes there.

– The person on kitchen duty (once in the morning and evening) will put dishes back to their place when they are dry (after everyone cleaned their own things) and makes sure that food is packed away correctly. Hanni will send you a calendar invitation for kitchen duty. In case you cannot do it, please find someone to replace you.

# Files and electronic storage

- The password for general lab databases is “Schuldiner”

- When you finish a screen, make a folder with all images from the screen, the screen guide and an Excel spreadsheet of the library coordinates, INSIDE the folder so that you always know which coordinates you used and have the details of the screen handy. The name of the folder in the microscope computer should contain the serial number of the screen (example SMS79), your name and date.  Confirm with Yeynit that the folder is compressed (with the SMS number) and that a copy is saved on the backup hard drive. Fill the names of the hard drives in the 'screen data base' Excel file in the storage column

- Optional: When you finish a screen, change the names of your files to the gene names with the special plugin that we have. This will make it easier to go back to data in the future.

- Have someone explain to you how to use the Excel function "vlookup" or “xlookup” – you will use it a lot! Make sure to use the “FALSE” value in the range_lookup parameter to receive the correct answer. In the newer versions of Excel there is an “XLookUp” option that allows for a specific output if value is not found (i.e., “N/A” or “Error”).

- Always update your electronic lab notebook (if you do not have one yet ask Maya to open one for you). Insert your thoughts about the experiment you already did and experiments you think about doing in the future. Enter DNA or protein gels results, pictures from the microscope, tables etc. Do this once a week (maybe set a specific day to do it) so that you won’t forget what you did as time passes.

- when updating your lab notebook make sure to write all details that you will need afterwards for publication – the exact methods you used (with dated protocol), the number of the strain, plasmids, primers that you used etc. If you are not sure ask someone in the lab which details go into thesis/publications later. You can also make a table for all antibodies that you used with dilutions etc.

- All information on libraries in the lab is in an Excel spreadsheet – please make sure to update any new libraries that you make or obtain, with all relevant info and also update on the sheet when you thaw your libraries.

- Make sure to update any information you have on strains, plasmids, primers and antibodies in the “comments” column in their respective spreadsheet, especially if you know something does not work or is not what it says – share this information with everyone.

- It is YOUR responsibility to make sure that all your data is backed up in at least one other place. It is your choice whether to do this on the cloud (DropboxinSync) or on an external hard disk (to buy an external hard disk ask Hadar/ Hanni). Make sure to back up everything regularly. Also, when you leave you MUST retain a compressed copy of all your lab files in your own possession, in addition to the copy that we will keep in the lab. If you move to live abroad, take it with you for any questions we may have on scientific files while you are not in Israel.

- Before using the hard disk: Check which formatting it has - if it is “FAT”, change it to “NTFS”! FAT is not good – some of our imaging files contain information that it cannot store and that part will be lost, and the file only be partially functional. Reformatting after data transfer will delete stored data, so do it before. Changing the format can easily be done using the built-in windows function – right-click on harddrive > Format etc.

If you have MAC, it is recommended to ask the IT for formatting so you will be able to use it on PC as well since Eva’s computer works only with PC hard disks. Alternatively, you could purchase a program called “Microsoft NTFS for Mac” by Paragon Software. It may be more reliable and stable (in case something happens to the HD) than the operating system the IT is formatting the HD to.

- When updating any files on the lab Dropbox in the shared documents, make sure that your Dropbox app is open and synced and that you have the most updated version of the file you want to edit. This will prevent conflicting copies of the file. When you’re done updating, close the file and check that it has been saved and updated in Dropbox and that a conflicting copy was not created.

- Use an external table to insert any information of new plasmid/yeast strain/new primers before copying it to the shared tables in the ‘shared lab folder’. This way the chances of a conflicting copy will be lower.

- Before submitting a manuscript from the lab, you will be required to provide ALL original data for each panel and each figure. It is advised to make sure in advance that you keep all the original data and label it in a way that is easy for you to find later. If you are not sure how to do this talk to Maya.

- We recommend NOT to call folders or files with a date 1_1_22 etc…It will be very hard for you to find it in the future and impossible for the lab to retrieve data if needed after you leave. Please consider calling files with a name that makes sense AND a date “Lec1PullDowns_1_1_22” for example…

- Sometimes, when doing a backup, the computer might say that the file name or path are too long and will not copy the file. Therefore, try and keep file names and paths to a reasonable size.

- It is highly recommended to take an “Illustrator” and “Photoshop” course at the WIS graphics department upon joining the lab. It will allow you to make better quality figures and save you a lot of time in the future. You will also get FGS Credit Points for it. Talk to Maya if you don’t know how to register for one.

- For computational work, image processing and analysis, we have an Excel file listing coded scripts and macros that are available in the lab. Feel free to browse through if there is something that is of use for you and give back by adding an entry if you created something.

- When dealing with large data sets or performing statistical analysis to your data, you might need the help of the institute’s statistician. His details: Dr. Ron Rotkopf +972-8-934-5162 ron.rotkopf@weizmann.ac.il

# Plasmids

- When getting a new plasmid (from abroad or from Yoav or if you made it) – always freeze it down as bacterial stocks in the -80 (using the NEBstable competent cells). Make sure to write the number on the top AND on the side and put a line under it to avoid it being read in the wrong orientation (like 508/805 and 9/6), so it can be identified easily. Please add a comment to it saying “never used”. Once someone uses it successfully for the first time, they should delete this comment.

- When ordering a plasmid from Yoav please make sure to go over the sequence and make sure it is exactly what you requested and the correct backbone.

- Whenever you freeze a plasmid make sure to save all relevant data in the folder of “Maps and Sequences” with the plasmid number clearly written on it.

- Please consider sequencing the entire plasmid from the frozen bacterial stock (in case that mutations happened in the bacterial transformation) – we now have an easy way of doing this through https://www.plasmidsaurus.com/. There is an institutional pickup box in the Meyer building on the second floor next to Rotem Sorek’s lab. Plasmidosauros picks it up every Monday and Wednesday. To order sequencing, please refer to the sequencing protocol: Dropbox folder, under “Protocols”->” Sequence a plasmid with SNPsaurus”.

# DNA Gels

- A DNA gel can be kept in liquid for a few hours after preparation but it is better not to use it after a night in liquid as the gel-red diffuses out and the signal becomes less clear. If you need to use it the next day, then you can wrap it in clingfilm (saran wrap) and store it in the fridge.

- Remove agarose gel from the plastic tray for imaging unless it is the original tray (with the rubbers in the edge); in that case either way works.

- Make sure to leave the sink and bottle clean after you pour your DNA gel leftovers. Clean the bottle with a brush.

- When done with the tray of the gel imager, always wipe with kimwipes (NOT paper towels so as not to leave small debris) to remove small pieces of gel and avoid the accumulation of salts from the dried TAE.

- After imaging, please transfer the images from the shared folder in Dropbox to your own folder, to free up space. The whole folder is 2GB and gets clogged up if you don’t move your images.

If the agar bits are really stuck, fill the bottle with water and heat it in the microwave to dissolve the bits. When preparing the agarose gel, it is very important to cool the gel down before adding and pouring the gel in the plastic tray, both to avoid Gel-Red fumes and to prevent disforming the plastic tray.

-It is sufficient to put max 3ul of ladder, no need for bigger amounts.

# Strains and Media

- W303 strains cannot grow on media with our OMM Amino Acid mix.

- When using G418/ Hygro in media it needs to be SD(MSG). MSG comes instead of ammonium sulfate so that the pH is more basic.

- The expiration of media: G418 and Hygro - 6 months, NAT/amp/Kan - 1 year, 5FOA/ MTX - 3 months, all the rest - until dry/contaminated

- Strains that you keep in your personal refrigerator should be refreshed once a month and thawed from the -80oC freezer at least once a year. Thaw your strains prior to starting an experiment (including a transformation).

- When you freeze down a new strain, use a printed sticker to put the strain number on the cap and also write it on the side of the tube with a pen - put a line under it to avoid it being read in the wrong orientation (like 508/805 and 9/6). Finally, paste a sticker with the strain number on top of the handwritten number on the cap and also print a round sticker and put it on the side as well (the writing serves as a backup system). Do not put rectangle stickers on the side as they can fall off.

- The lab has several BY4741 and BY4743 strains from different sources.  Make sure you pick the best one for your experiment (the one that is “negative” for your phenotype) and write down the strain number in your notebook so that, if necessary, you can repeat the experiment with the same strain.

- Note that yeasts behave differently when they express the various auxotrophy markers (Leu, His, Ura, Met etc.) and even if they have an antibiotic resistance marker (NatR or KanR or HygroR) integrated into it – so use the correct strains as controls for your experiments with the same marker integrated into their genome.

# Transformations

- You can make the PCR amplification for transformations ahead, to ease the workload on the day of transformation.

- You can keep PCR amplification products in -20°C for extended periods of time and get back to them if you need to repeat the transformation.

-  If you encounter problems when trying to amplify a PCR cassette for transformation using Kapa Hifi polymerase (could be that you get 2 bands in the gel after running the cassette), consider changing the PCR program - longer time or more repeats for specific steps can increase the efficiency, as well as changing the Ta or adding 1ul of DMSO. Or try using a higher-fidelity polymerase such as Q5.

- If you want to do a check PCR on a strain with a plasmid (when the plasmid expresses a protein without a fluorophore) you should do it with KapaHifi and not GoTaq. Another option is to check for the presence of the amp resistance part and use GoTaq.

- Before you order primers check if we have them in the database by the sequence, and not by name/gene. Sometimes people order the same primers under different names. It saves time and money. Also check the SWAT pYM primer collection (All SWAT pYM Excel sheet – there are pYM N’ tagging (F and R) and N’tag CHK R primers.xlsx) which can be found in for almost EVERY GENE here! Sometimes the WT CHK R is the same folder as the N’tag CHK R, so do check it here too.

- Primers that were picked from the SWAT collection should be moved to a screw-cap tube (to reduce liquid loss) and given a number to be added to the general primer collection.

-If you are taking primers from the SWAT collection and notice the cover is a plastic one, then please spin down the plate and replace it with an aluminum cover. Do not pierce the plastic cover and repair it with a piece of aluminum.

- It is recommended to make sure that the primers fit your needs by looking at the plasmid map and sequence before ordering. It will save you time and effort in the future.

- When you finish a primer in the common stock, order a new one and give it the same number. Do not add it as a ‘new’ primer to avoid duplications. If the concentration of the primer changed (since we now use 100mM) – update this information in the Excel!

-if you are ordering primers with value 1000 shekels or more, let Hanni know so she can make sure we have enough credit to cover it

- After transformation: it is better to wait at least three days until all the negative colonies stop growing and the positives become clearer. We sometimes hurry this up and replicate/velvet without good reason.

- After transformation – ALWAYS streak to singles even if something looks like it is one colony. You never know if the colony is really genetically homogenous and this could make your life very difficult later on if it is not. Only after streaking to singles take a colony for Check PCR and the same colony for growing in liquid and freezing etc.

- If you transformed something that has antibiotics selection and you do not have time to wait until the recovery time is over (1 hour for G418 and Hygro, 3 hours for NAT), you can plate the transformed yeast on YPD plates and replicate using velvets onto selection containing plates the next day. You can also leave the cells to grow in liquid YPD and then plate with selection the next day.

- If you need to use an expression plasmid to mark an organelle, consult the file for organelle markers in our shared Dropbox.

- If you want to simply replace a selection marker in your strain, you can do that using designated plasmids in our plasmid list or using primers that will amplify a different selection from a plasmid of choice, where the promoter and terminator are the same as in your strain. This way, there will be a recombination in the promoter and terminator areas flanking the resistance and you will be able to swap it.

- It is very helpful to keep a good and clear file of all your transformations, so you can easily find what you did and with which primers (see suggestion below)

- When creating a strain with a contact site reporter using the "split-Venus” method, test both proteins of the contact site reporter, with the two parts of the Venus proteins (VC and VN). In some cases, you can get a better signal with one pair over the other.

- The BY4741 strain has only partial deletion of his3 gene and so revertants can come up. Therefore, HIS is not the most reliable selection. If possible, choose another selection.

- Before doing a transformation - think about the figure you want to show in the end and make sure you have all the right controls (for example - maybe you should also transform something into BY)

- Before performing any genomic modification to a gene, check in SGD if it is essential, if it has close homologs or if it’s overlapping another gene on the opposite strand. If the gene is essential and you still want to ‘delete’ it – consider adding Gal promotor and raise on S-Gal plates until experiment time.

- You can use Blat (https://genome.ucsc.edu/cgi-bin/hgBlat) to check if a sequence of choice (gene, primer binding site) has multiple appearances in the genome. Make sure to choose S. cerevisiae genome for the search.

-When planning Gal-promoter swap transformations think if the transformants would prefer being plated on Gal plates or Raf plates, rather than glucose plates (which would be, in essence, making deletions).

- When making deletions of mitochondrial proteins that might have a respiration phenotype, pick the smallest colonies to do check PCR (the big ones will be wrong) and consider plating on 4% glucose after transformation

- For really sensitive strains you can do a “soft” transformation – 30’ in 30ºC and 15’ in 42ºC. If cells are super sensitive, you can wash them in 10%LiAc buffer instead of DDW, before adding transformation mix.

- If your transformation does not work twice consider re-ordering the primers. Sometimes even one point mutation in a primer inhibits homologous recombination and this occurs quite frequently!

- If your transformation still does not work after you have ordered new primers, consider doing it in a diploid (BY4743). This way, if the change to the genome was lethal, it will be covered by the second copy, and you can then see that was the reason.

- If a certain CHK PCR does not work repeatedly, make sure you allow enough annealing and extension time in your PCR program. If still does not work- use Kappa Hifi instead of GoTaq.

- Always make sure that you know which ladder you are using and check that your PCR product is the size that you thought it would be. Primer dimers run at the bottom of the gel (often below the smallest ladder marker) and often look like a PCR product, but they are not.

- It is crucial to check a strain that has a visual phenotype with both check PCR AND microscopy before freezing - sometimes the positive colonies in the check PCR will show a different phenotype in the microscope than expected.

- Before starting the transformation – make sure you have all the plates you need (type and quantity) for your plan (if you need YPD plates for example).

- Use positive and negative controls in your PCR to make sure you have taken the wanted product.

-You can back dilute straight to 15ml tubes and put in the rotor in the incubator under the autoclave in the hall.

# General yeast growth

- If you have a long experiment and you want to start early with yeast in log phase, you can dilute in the evening and grow it overnight to hit mid-log in the morning. There is a calculator in the Dropbox. For that you should put starters on in the morning a day before, so you’ll be able to dilute it in the evening.

- You can leave yeast starters on the bench over the weekend, they will be ready to be back diluted on Sunday. Similarly, you can leave yeast agar plates at room temperature (in a drawer or in a box, not sealed completely to allow air flow) and colonies will be ready in two days.

- If you plan to do protein/RNA extraction from cells, you can gently spin down the cells, wash once and snap freeze the pellets in LN2/dry ice-EtOH bath. These can be stored in -80 until you are ready to process them.

- Clean your fridge ~ 1x a month, to see if you have spare plates and verify you have replicated everything that needs to be refreshed or should be thrown away. Your spare plates can be placed in the general fridge at the ‘spare plates’ box. Do not forget to fill the information about these plates so they can be used by other lab members.

- For measuring OD make sure you are reading in the linear range, 0.1-0.5 (in 0.6 it goes down to 93% accuracy and then further). This will be approx. 20 microliters from the O/N culture diluted in 980 microliters of DDW/YPD or 500 microliter of midlog sample in 500 microliter of DDW/YPD (can also be 250 into 750).

- Glass tubes can be used directly in the OD reader. This is particularly useful so as not to waste any volume of your precious culture when growing cells to mid-log phase. Simply use another glass tube with the same media as your blank (3ml are sufficient for the light beam to cross through the liquid). Do not forget to vortex your tube before measuring OD.

- Take into consideration that different strains grow differently, so if you are planning an experiment that relies on the cells to grow in a certain time (for example – B.D in transformation or westerns) – you may have to change plans accordingly.

# Singer plates and pads

- When you finish your work on the Singer, put the dirty pads IMMEDIATELY in bleach. When the yeast dries on the pads they act like glue which makes it difficult to wash them properly. Also, make sure that there is no paper stuck to the bottom pad. When going from liquid to Agar it is best to use new pads.

- Place dirty Singer pads one by one in the washing boxes in 4 piles (not 3), fill a box before starting a new one (but make sure not to overfill causing pads to break), fill up with just enough 10% bleach to cover the pads (no need for them to be swimming and floating around in bleach). Make sure to press any air bubbles out from between pads. Full boxes go in the top drawers of the cabinet by the Singer and will then be taken for recycling in the 3rd floor autoclave room.

- After you take your pads out from the Singer’s dirty pads’ drawer, replace the paper lining with new paper (paper disposer is hanging on the wall next to the Singer).

- Do not recycle Singer plates if they are highly contaminated and the fungus touches the plate itself and/or the contamination is “fluffy” (these are spores that will disperse in the lab the minute you open the plate) - throw them to the biohazard bin.

- For short pads (96, 384 short pads and 1536 pads), add plastic dividers so that the 4 piles stay organized, and remove the dividers before recycling.

- Before using pads and during pad recycling duty, please make sure that the pins of the pad are not broken or crooked. If they are, dispose of them to the biohazard bin or, if they are sterile, in the plastic recycling.

- It is always good to double-check that your pads are completely dry before loading them in the Singer-hopper. This will save you a lot of pain!

- After touching dirty pads and before continuing using the Singer – change gloves.

- Make sure to label the plates beforehand to avoid confusion. If using stickers – print stickers before.

- Put the stickers on the plates without lifting the lid. When the sticker is placed below the rim of the lid, it is tricky to peel it off during lab cleaning.

- If you notice that a Singer plate is not completely intact, dispose of it in the lab plastic recycling bin (black lid). Sometimes even a small chip on the rims of the plate may chip when the Singer put the lid back on, and the little chip shoots inside the plate and mixes your colonies.

- After replicating plates in the Singer, always make sure that the plate and the pattern was replicated correctly.

# SGA & Libraries

- When planning an SGA and ordering plates, remember that there are plates that you can take from the general stock or from our “spare plates” stock. If you need plates from the general stock, take them in advance and put them in your fridge, otherwise they might no longer be there when you need them. Make sure to order more for the general stock if needed.

-When ordering plates for SGA, order about 20% more than what is needed to account for pinning errors on the Singer.

- Keep all the plates from the SGA until you finish the whole process - you might need to go back and check patterns in the plates or start from the diploid selection.

- Keep SPO plates outside the fridge after replicating to haploid selection - sporulation is generally very inefficient and you might have to return to the plates a few days later to get more spores. Additionally, you can also pin several times with different diameters (manually or using one of the Singer options) in the SPO plate to get a higher number of spores for each colony.

- If you are picking strains from libraries, then also ALWAYS streak to singles, since colonies can easily get contaminated with cells from close-by colonies. Perform a check PCR to ensure you are using the correct strain.

- When using the Singer, please alert Yeynit or Hadar immediately if anything is not working perfectly as this means that the Singer has to be serviced. For example, if the pad is not touching colonies = time to clean the pad head/check offset hasn't been changed.

- While replicating on the Singer, if the corners of your plate do not replicate well, reduce the pressure of the pad gripper or use the mix option or elevate the edge of the plate with a small piece of tape (given that the agar is leveled).

- For optimal Singer plates, if you pour your own plates, make sure not to pile them higher than 5 plates to let them dry as leveled as possible

- When replicating from agar to liquid for imaging, there’s no need for liquid mix. It is also better to use lower pressure (30%), it gives a more homogenous replica.

- Read the “SGA working protocol” before planning (all plans needed are detailed there). There is also a video (made by Ofir) that explains the basics of SGA, how libraries are made, and how to work with them properly. It’s in the lab shared folder under “protocols” (the only video)

- If you want to make sure that your SGA will not be delayed, you can prepare and replicate 2 copies of SPO in advance (each of them in a different location) when moving from diploid selection. This way if the first copy does not grow well/gets contaminated on Haploid Selection after 5 days, you can use the second copy a few days later. Going back to a used SPO plate does not give good results….

- When taking libraries from the general stock, make sure to take the correct copy (picking or replica). DO NOT take the ‘DO NOT TOUCH’ copy. If there’s a problem with the library, alert the person in charge of it.

- If you are in charge of a library, you can use the “library cards” aid (the file is found in the Libraries folder, in “Aids and tools for library work") when refreshing it monthly. This aid creates a printable pattern of each plate and will allow you to easily track missing colonies and save them.

-After you finish your SGA and before screening, make sure to fill the ‘Screen guide’ and do all the necessary checks. Send the file to Maya and Yeynit. You will have to get approval for starting the screen and a number for the screen from Yeynit. This is also relevant to screening small amounts of plates.

-If you plan to change steps in the robot screen operation or you need a new protocol on the robot for screening - please notify Yeynit as soon as you start planning the SGA. Yeynit needs enough time to write the programs/ change them and run test runs before you can run your screen and this may take time.

-  If you need to thaw a single strain from a library you must get permission from Maya to do this. In general, taking out single strains is not allowed as it can cause them to disappear from a plate when the whole plate is thawed.

-We reuse 384-well plate lids for screens: before placing ON plates in Eva incubator- clean the lids thoroughly with 70% ethanol (100% can be used for faster drying). Repeat the same cleaning in the next morning for the BK plate lids.

# Western Blots

- Please dilute all your antibodies in 2% BSA/PBS + phenol red. You will find this in the fridge. If it runs out, please notify whoever is in charge to make more.

- All diluted primary antibodies that we have in the lab are kept in one fridge – please check which. Diluted antibodies should be kept until the red solution turns yellow. ‘In use’ secondary antibodies are in the common fridge. Diluted antibodies located in the fridge will be discarded after 1 year/turn yellow, if they are commercially available. Antibodies that we get from other labs/discontinued are stored either way.

- When diluting primary antibodies, you MUST write the information on the side – Serial number, dilution, date (If you want to add more - enjoy) and at least the serial antibody number on the cap.

- A list of antibodies available in the lab can be found in the shared Dropbox folder along with indications where to find it in the antibody freezer (‘working stock’ and secondary boxes for respective aliquots in use, ‘DO NOT TOUCH’ boxes for stored aliquots). The labeling on the aliquoted tubes refers to the fluorescent channel used for detection with our LICOR (700 or 800). For example, an antibody originally called after its emission maximum of 680, then is listed as 700 for simplification. If info about the specific identity of an antibody & manufacturer is missing in the list, please ask Olga (guardian of our antibodies).

-When an antibody arrives, it needs to be aliquoted (usually to 10ul aliquots but it depends on the antibody), labeled properly, the aliquots stored in ‘working stock’, the original vial stored in ‘DO NOT TOUCH’ box with some indication on it that it’s empty. This is the responsibility of whoever ordered it/used it first. If you have any problems or questions – talk to Olga.

When you take an aliquot, keep that for yourself and use it until it’s finished. The antibody vial from the company/collaborator (which contains a few microliters) will be stored in the main box (‘DO NOT TOUCH’) and should not be used unless in an emergency (aka – someone used all the aliquots in the ‘working stock’ box and the re-ordered Ab has not yet arrived). It is important to keep the company vial for reference in case anything was entered incorrectly onto our spreadsheet.  – do not throw away any vial without checking with Olga first.

- When you decide on a primary antibody, be aware that it might not/only work against the yeast or mammalian protein. This does not apply to secondary antibodies which only need to be matched to the primary antibody (raised in mouse or rabbit) and not to the organism (yeast or mammalian cells). You can check in this website https://www.citeab.com/ if the antibody has been used before and for which applications.

- Milk contains lots of biotinylated and phosphorylated proteins, so do not use this for streptavidin blots or anti-phospho antibodies. Instead, we have Fish Serum blocking solution in the lab (white bottle, 1:5 in PBS, 4oC fridge in the hall).

- The anti-histone H3 antibody (#59) is commonly used in the lab as a loading control. It is a rabbit polyclonal, and the detected band runs at ~17kD, so rarely interferes with your protein of interest. If it does, or you do need an anti-mouse loading control, then you can try our Hsp70 (#72) antibody. A new anti-Actin antibody can also be used (#191). Or the mouse anti GAPDH (#217). Fr the GAPDH it is better to use in RT for 1 hour, since it doesn’t do well in the cold room.

-To reduce the size of the membrane to fit the box better, you can cut the borders of the membrane after Ponceau staining.

-It is recommended not to cut your membrane in your first trial of running the samples, you might miss some bands that you did not expect.

- It is essential to run a control sample (e.g. BY4741) to check specificity of the antibody. There might be times that a band is not specific to your samples and can also be found in the control lane.

- Pre-cast gels are to be used ONLY for publication quality images or if gradient gels are required. For all other uses, please pour your own gels or ask special permission from Maya.

- Pre-cast gels do not always work with RIPA buffer. Avoid RIPA if possible or test it before doing a big experiment.

- Be aware that some buffers interfere with accurate, quantitative running of BioRad pre-cast gels. Bands may (or may not) appear at the right molecular weight but look heterogeneous, without really being different, so it is a tricky artefact – not quantifiable and not pretty. (BioRad does not determine which buffers but be cautious with RIPA buffer, commonly used mammalian cell lysis.)

- Do not forget to remove the green tape from the bottom of pre-cast gels before loading your samples.

- To look at proteins of small molecular weight (10kDa and smaller) that would normally run at the front of a regular 15% gel, you can prepare instead a 19% “UREA gel”. 10 kDa will appear in the center of the gel! And everything above 70kDa will most probably not look good and be squeezed. The Prestained protein ladder works fine, although it reveals an additional band between 20-40kDa (don’t be surprised!) The recipe includes Urea and a higher ion concentration so a few things to consider: i) heat generation above 0.3A, means running at 80V works fine, ii) precipitation in the cold, so store not in fridge but in a wet chamber to prevent drying (lasts a week or two, then let the gel soak and rehydrate in the blot chamber), iii) Weigh Urea powder according to the new recipe and wait until completely dissolved, iv) use 2M Tris buffer stock for the separation gel. Other reagents stay the same, just used in different amounts, see recipe.

- Antibodies in sodium azide can be discarded in the sink. In case the azide concentration is higher than 0.05%, ask safety for guidance - Safety Unit (phone: 2950 or 3457). The antibodies that are diluted into our ‘red solution’ are safe to be discarded into the sink (the azide in the solution is diluted enough).

- If you need to, you can leave your membrane in blocking solution or primary antibody (if you know it produced a clean signal after overnight incubation) over the weekend, shaking at 4deg. However, never do this with the secondary antibody.

- If you are doing an IP followed by WB, then avoid using the same species Abs for IP and WB as you will only ever detect the heavy and light chain bands. If there is no option and you have to use the same species, then if your protein of interest is significantly above 25kD, you can run the gel and transfer as normal, then cut the lower part of the membrane (just above 25kD) and detect your band using a light-chain-specific secondary Ab.

- For western blot quantification you should always check the total protein amount in your sample using BCA (or another method). The loading control, Ponceau or the pre-lysis OD is not enough.

- When preforming BCA – dilute your samples by 4/5 and always make 2ul extra - in the protocol it’s 25ul per well but works great with a 1:5 dilution and 15ul per well. Whatever you choose, remember to add the same amount of the standards as your samples.

- If you need to quantify your bands - in the protocol folder there is a protocol for how to best quantify and which program to use

- The NaOH method can be used only for qualitative assessment because you cannot quantify the total protein level.

- The NaOH method can only be used for soluble proteins as it requires boiling which is incompatible with membrane proteins.

- Do not boil membrane proteins, they tend to aggregate. Instead put them at 45oC (or up to 75 if needed) for 15m.

- For proteins above 175kDa, do not use the semi-dry transfer but only the wet transfer. The semi-dry will work but will not be quantitative.

- Bromophenol blue (in the standard sample buffer) is highly fluorescent in the 680 channel, orange G is a recommended alternative to reduce background.

- If you have many proteins to look at and you decide to cut your membrane for incubation with antibodies, you may like using 50ml falcons (swirl the membrane on the falcon wall if the panel is very long) and our two new tube rollers (one in the cold room and one inside the lab at room temperature). It saves resources (less than 10ml of antibody solution required), saves you from looking for tons of other boxes and allows you to conveniently store your antibody solution afterwards in the falcon.

- Before starting the protocol – make sure you know the relevant type of the first antibody (mouse/rabbit) and make sure to use the correct secondary antibody.

-If needed, instead of the Extra Thick Blot Filter Paper that we use to transfer the gels, you can use 3-5 layers of regular Whatman paper (which you need to cut to the relevant size).

- If you want to be extra resourceful in saving antibody or you prefer to store your Western Blot wet (re-decoration of dried membranes can be tricky), you may like using home-made plastic bags for incubation and storage of your Western Blot (complete or cut). To do so, we use our little heating machine, next to the laminating machine in front of Maya’s office. It melts and seals the plastic foil into a bag of every size you need, matching the size of your membrane. Usage of 3ml antibody solution is sufficient. You may go even lower.

# Microscopy

- When going to the microscope make sure to take all the correct controls. For all fluorescence measurements you must take a strain with NO fluorophore as background (you will be surprised but yeast have weird auto fluorescent organelles that change in different conditions). For co-localization studies you MUST take strains with each protein tagged individually to check for bleedthrough of the fluorophores. Bleedthrough must be measured separately in each experiment - while there might not be a bleedthrough in one experiment, there might still be in another even though the microscopy settings are the same, because of a protein that is more abundant or with a more concentrated location in the cell which leads to a stronger signal.

- It it important to take an image in brightfield (BF) for future quantifications (that utilize cell segmentation), and to demonstrate the presence of cells and their outlines, in case of no fluorescent signal. When preparing figures for publication, instead of showing the BF, you can also highlight the cell surrounding as dotted white line in the (black) fluorescent picture.

- ConA plates are viable for three months. After three months from conA addition, use with caution.

- When repeating an experiment using the same microscopy system, load the settings from the previous experiment - this will save you time and make your experiments more comparable if you need to do quantifications in the future (.xml file in ScanR and .oex file in CellSens – does not include saving of the intensity of light source.).

- When imaging samples of multi-colors start with exposure of the long(er) wavelength(s) fluorophores or sensitive, less photostable fluorophores, to avoid bleaching of the others.

- Please use our lab spinning disk Microscope (connected to Eva) only in cases where no other microscope can be used such as the use for the double camera, the Sora, the incubator, ScanR analysis etc… In all other cases please use the departmental spinning disk.

- When running microscope experiments make sure you take enough fields to make quantifications/statistical analysis if needed.

-When going over microscopy images from a screen for the first time, ask for guidance on how to do it optimally. Methods may vary between experiments. Beginnings are hard and take time, but if you are going over images and haven’t finished going over a plate in the course of 2-3 days- ask for help, it shouldn’t take this long. Generally, you should be able to go over several plates per day.

- After using the oil objective, clean it from oil with 100% ethanol or isopropanol using the designated lens paper. Also look for oil drops on the floor. Label this plate that you know to use it only for oil imaging in the future.

- Keep separate 384 imaging plates for oil and air.

- For turning on the EVA microscope: click the main power, wait for start-up noises to finish, turn on the RTC controller (upper shelf) -wait for the two beeps before continuing (the first beep is immediate and strong and the second takes about a minute). Turn on the lasers, wait for a light of “Laser Ready” to appear on the black keys. Then turn on the TPC (little screen on the drawer, to the right of the big PC screen), before opening the software. And the other way around for turning off EVA: first turn off the TPC, then the controllers, then the microscope. While usually always “on”, over the weekend, the tango box (one of the boxes on the shelf) might have been turned off. So, if there are problems, check if it is on and properly communicating (blue light, switch is on the back, not the front as for the other boxes).

- CellSens offers personalized layouts that allow you to individually arrange all the windows for the different settings the way you want. This just affects the way it is displayed on the screen. Other changes you make will also affect other users and vice versa. Of note, the “operation methods” (that you can save and which then appear as buttons labeled after the channel settings) only save a few hardware settings, such as light source and filter set, and not the intensity or exposure time.

For independent users, before imaging, best check the following: i) settings for saving (select the info you want to save in the name of every image, such as WellID or PositionID, the order of numbering, directory, mark the box if images should be closed after saving or not), ii) channel intensities, iii) exposure time, iv) check that the position list is empty and not filled with old entries. The stage may otherwise move to unreasonable places.

CellSens offers to save separate files for: i) the experiment - imaging workflow (.oex), ii) the actual image, which can be displayed in ImageJ using an Olympus plugin, or be exported in CellSens as ome.tif (.vsi).

-To open a user and get access to the departmental spinning disc microscope, set a training session with Yeynit. To use any other of the fluorescent microscopes in the molecular genetics department (Belfer building, left side: “LM900 point scanner” also for Airyscan, right side: “Dragonfly” for Immunofluorescence), talk to Eyal Schejter.

-For analyzing images from the departmental spinning disk the images must be in scanR, you will have to convert them to scanR format (use scanR Analysis software to convert). In addition, you need to notice the order of the wells you imaged because the file names of the images will follow this order. However, in the .nd file, every serial name has the well description.

-To add the well description to the serial image name from the spinning disk, download the software splitToWells.exe  from the Dropbox.

- For analyzing images, we have a Wacom tablet that you can connect to your computer to use it with ImageJ etc... It makes it easier to draw lines and follow structures to measure all kinds of parameters.

- For visualizing and processing microscopy data, consider learning and using ImageJ macros. By using code to automate the processing, you can process images much more efficiently and robustly. There are macros people wrote in the past in the shared “Software” folder.

Troubleshooting EVA microscope (FOR INDEPENDENT USERS):

Although it might be tricky to provide an absolute solution to the problem you may observe during a microscopy session with EVA, check the following actions that were useful previously. Some of these can be carried out without worrying about causing harm, others you may want to do only after having tried other things first, some might mean a bigger interference with the microscope, so ALWAYS TALK FIRST WITH YEYNIT for briefing, also as an independent user.

| Problem | Solution |
| --- | --- |
| Connectivity (mic with software) | - Close the software and turn off/on the TPC (little screen on the drawer, left of the big PC screen) |
|  | - Check the Tango box. If it is on, maybe turn off/on. A blue light signals proper connectivity. |
|  | - Turn off/on the microscope (wait until the two beeps of the RTC (As above – one strong and immediate, the second softer and after a minute) controller - upper shelf - before turning on the others) |
|  | - Turn off/on the complete computer (may sound trivial, yes). |
| Focus | Clean the plate AND the objective (oil residues, dust particles and moisture may hinder focusing |
|  | Check the limits of the z-level set in Cellsens (for yeast plates: 6335µm; for EM grid holder: 9800µm) |
|  | Use a fresh plate (96-well plates tend to cause problems when stored and used another time, if there were free vials). -If you use the incubator at 37C, discard your 384 well plate afterwards since the heat tends to affect the glass of the plate and it’s harder to focus after a while. |
|  | See above, turn off/on devices… |
|  | Lost positioning of stage holder for plates (ASK YEYNIT). (Requires opening of screws – at A-position first - to push stage holder back to its intended position – happened to be 0.5mm off, hard to notice) |
|  | Turn on & pre-heat the microscope 20 min in advance (for SORA experiments and regular ScanR) |
| CellSens craziness (mismatch of imaging & settings – images look like plain colored paper; or any other non-smooth, weird imaging (unusually slow or dim). | ‘ScanR-bypass’: Close CellSens/ Open & close ScanR/ Re-open CellSens and image. (Accurate explanation can be given by Yeynit, though all you need to know is do the ‘ScanR-bypass’. Roughly, ScanR can ‘restore’ settings due to some capacity to communicate with EVA mic that CellSens is lacking, according to Olympus technicians.) |

# Orders

- Please add your product name, catalogue number and supplier to the file entitled: “lab ordering list” located in “Life in the Schuldiner lab”/ “orders”/ “lab ordering list”.

- Make sure to add your new order name in black. Orders that appear in gray have already been ordered by Hanni (but haven’t been received yet).

- Please make sure to get an updated price quotation for your product and file the quote in the folder entitled: “orders”.

- Every week a list of the items that have been ordered by Hanni will be hung on the side of the pipette cabinet in the lab. Please update this list whenever you get any delivery, and if the item is not in the list, then add it manually. Write your name, the location of the product and add a comment when necessary.

- Place all the documents attached to the deliveries (such as invoices/delivery notes) on Hanni’s desk.

- A worksheet including all our ongoing ordered items has been added to the “lab ordering list” file. Please add to this list stuff that is ordered on a weekly/monthly basis so that it will be easier to find the details.

- When you receive an incoming order (brought by delivery people or placed behind Terry’s desk next to the TC room), please do the following three things: 1) If it’s clearly stated, store the item in the designated place in the lab or at -4°C/-20°C (“Inbox” shelf in the common lab fridge and freezer). If not clear what the storage conditions are – put in the 4°C fridge in the hall and send a picture of it to the relevant person who will be responsible to relocate it to the correct storage conditions, 2) bring the paperwork to Hanni’s desk, 3) indicate that it has arrived on the print-out of the lab ordering list hanging on the central shelf where we store our pipette tips (if you can’t find the item on the list, please add it. The list is renewed once a week). 4) If the order is designated to a specific person in the lab, please send them a message notifying them of its arrival so that they know.